# Synergies and trade-offs between climate-change mitigation and adaptation

Industry and transport — an internal literature note for the Policy Gap 2.0 prep cycle

Where cutting emissions also builds climate resilience, and where it works against it — mapped subsector by subsector, on the report's own structure.

Summer Prep · Policy Gap 2.0 Report · Note 2 — background document, prepared 27 July 2026.

Status of this note Each entry states a real, literature-grounded interaction and cites the primary work it rests on (IPCC AR6 WGII/WGIII, the EEA European Climate Risk Assessment, and peer-reviewed studies). Every citation was source-verified against Crossref and publisher records on 10 July 2026. The framing, and the assignment of each mechanism to a subsector, are working judgements compiled for this prep cycle and are pending sector-lead sign-off. Nothing in this note is a Board position.

# Why this note

Mitigation and adaptation are handled in separate policy tracks, but in industry and transport they meet in the same assets: the same plant, the same corridor, the same fleet. This note maps where they reinforce each other and where they pull apart.

What the note does

It takes each subsector of the 2024 progress report's industry (Ch. 5) and transport (Ch. 6) chapters and asks two questions of the mitigation levers in it: does this lever also reduce climate vulnerability, and does any adaptation need in this subsector push emissions the other way? Each interaction is recorded with the mechanism behind it, the implication for EU policy design, and the literature it rests on.

How to read an entry

Every entry is one table. It names the mitigation lever and the adaptation dimension it interacts with, sets out the mechanism connecting them, and closes with what follows for policy design. The tag in the first row says which of three directions the interaction runs:

- Synergy — The mitigation action also reduces climate vulnerability (a win–win).

- Trade-off — The mitigation action increases climate vulnerability, or an adaptation need raises emissions.

- Conditional — The interaction is a synergy or a trade-off depending on design, siting or resource context.

The subsector taxonomy deliberately mirrors the report, so this note and the report's own chapters stay comparable — and so a finding here can be carried straight into the corresponding chapter section.

What it is not

It is not a quantification: the entries say which way an interaction runs and why, not how large it is. Where the literature is thin or the sign depends on design choices, the entry is tagged conditional rather than resolved in one direction. And it is not a Board position — it is preparatory material for the sector leads.

# The map at a glance

How the interactions distribute across the two sectors and their subsectors.

Table 1	Interactions per subsector and direction

| Subsector | Synergies | Trade-offs | Conditional |
| --- | --- | --- | --- |
| Industry |  |  |  |
| Iron & steel | 1 | 1 | — |
| Cement & lime | — | 2 | 1 |
| Chemicals & petrochemicals | — | 1 | 1 |
| Cross-cutting — circularity & material efficiency | 1 | — | 1 |
| Transport |  |  |  |
| Road passenger (cars & vans) | 1 | 1 | 1 |
| Road freight (heavy-duty) | — | — | 1 |
| Rail | — | 1 | — |
| Aviation | — | 2 | — |
| Maritime & inland waterways | — | 2 | — |
| Cross-cutting — demand & modal shift | 1 | 1 | — |
| All subsectors | 4 | 11 | 5 |

Notes: counts are entries in this note, not a measure of how important each interaction is — a subsector with one well-evidenced trade-off may matter more than one with three synergies.

Source: this note; subsector taxonomy from ESABCC (2024) “Towards EU climate neutrality: Progress, policy gaps and opportunities”, January 2024.

# Industry

Iron and steel, cement and lime, chemicals, and the circular-economy layer that cuts across them — where deep decarbonisation changes an industrial site's exposure to climate risk, in both directions.

## Iron & steel

Table 2	Scrap-based electric steel cuts emissions and supply-chain exposure

| Synergy | Scrap-based electric steel cuts emissions and supply-chain exposure |
| --- | --- |
| Mitigation lever | Shift from blast-furnace/BOF to scrap-fed electric-arc-furnace (EAF) steel. |
| Adaptation dimension | Lower dependence on imported coking coal and iron ore, and lower process-water intensity. |
| Mechanism | Secondary (scrap-based) steelmaking cuts total (direct + indirect) CO₂ by roughly 85–90 % versus the integrated route where the electricity is largely decarbonised — the IEA roadmap's 2.2 vs 0.3 tCO₂/t pair is direct + indirect, not direct-only (direct-only is 1.2 vs 0.04) while also shortening and localising the supply chain, reducing exposure to climate- and geopolitics-driven disruption of coke and ore imports. (A water-intensity advantage for EAF over integrated works is often asserted but is not supported by the sources cited here — the IEA roadmap gives only a sector-wide average and does not split by route.) |
| What it implies for policy | Circular-economy and scrap-quality policy (CEAP, Ecodesign) is simultaneously a resilience instrument; the report already flags the EU’s recycling-first, prevention-last gap for industry. |
| Literature | - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ - IEA (2020) "Iron and Steel Technology Roadmap" (direct + indirect intensities ≈2.2 tCO₂/t BF-BOF vs ≈0.3 tCO₂/t scrap-EAF; direct-only ≈1.2 vs ≈0.04). https://www.iea.org/reports/iron-and-steel-technology-roadmap - Material Economics (2018) "The Circular Economy — A Powerful Force for Climate Mitigation", Material Economics/Sitra. https://www.sitra.fi/wp-content/uploads/2018/06/the-circular-economy-a-powerful-force-for-climate-mitigation.pdf |

Table 3	Green-hydrogen DRI is water-hungry in water-stressed regions

| Trade-off | Green-hydrogen DRI is water-hungry in water-stressed regions |
| --- | --- |
| Mitigation lever | Hydrogen direct-reduced iron (H₂-DRI) powered by renewable electrolysis. |
| Adaptation dimension | Freshwater availability under more frequent and severe droughts. |
| Mechanism | Water electrolysis needs ~9 litres of purified water per kg of H₂ stoichiometrically — on the order of 20–30 litres once water treatment is counted, plus cooling water on top (Tonelli et al. 2023). Siting large H₂-DRI plants in southern/Mediterranean regions that are attractive for cheap solar power can collide with declining summer water availability, creating a maladaptation risk unless desalination or reuse is planned in. |
| What it implies for policy | Green-steel siting and Net-Zero Industry Act support should carry a water-stress screen; the EUCRA rates southern-European water scarcity as a high and rising risk. |
| Literature | - Tonelli, D. et al. (2023) "Global land and water limits to electrolytic hydrogen production using wind and solar resources", Nature Communications 14:5532. https://doi.org/10.1038/s41467-023-41107-x - Vogl, V., Åhman, M. & Nilsson, L.J. (2018) "Assessment of hydrogen direct reduction for fossil-free steelmaking", Journal of Cleaner Production 203:736–745. https://doi.org/10.1016/j.jclepro.2018.08.279 - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment |

## Cement & lime

Table 4	Building climate defences drives cement demand

| Trade-off | Building climate defences drives cement demand |
| --- | --- |
| Mitigation lever | Decarbonising and moderating clinker/cement output. |
| Adaptation dimension | Concrete-intensive protective infrastructure — dykes, sea walls, flood defences, reinforced foundations. |
| Mechanism | Hard adaptation infrastructure is overwhelmingly concrete- and cement-based, so ramping up coastal and flood protection pushes cement demand — and its hard-to-abate process emissions — in the opposite direction to the mitigation target. The larger the unmet adaptation deficit, the stronger the counter-pull on the cement budget. |
| What it implies for policy | Adaptation investment plans and cement decarbonisation (CCS, clinker substitution) must be budgeted together; nature-based and low-clinker defences reduce the clash. |
| Literature | - Habert, G. et al. (2020) "Environmental impacts and decarbonization strategies in the cement and concrete industries", Nature Reviews Earth & Environment 1:559–573. https://doi.org/10.1038/s43017-020-0093-3 - IPCC (2022) AR6 WGII, "Impacts, Adaptation and Vulnerability" — Ch. 13 (Europe) & Ch. 18 (Climate Resilient Development Pathways). https://www.ipcc.ch/report/ar6/wg2/ |

Table 5	Lower-carbon, durable and reflective concretes aid heat resilience

| Conditional | Lower-carbon, durable and reflective concretes aid heat resilience |
| --- | --- |
| Mitigation lever | Clinker substitution, CO₂-cured and novel-binder concretes. |
| Adaptation dimension | Longer-lived infrastructure and cooler urban surfaces (reduced urban-heat-island effect). |
| Mechanism | Blended and carbonation-cured concretes cut embodied carbon, but the durability case is conditional and cuts both ways. Habert et al. find that carbonation can damage the electrochemical protection of steel reinforcement and is deleterious for the durability of concrete exposed to rain or high humidity — precisely the climate-exposed infrastructure at issue here — while noting that more than 80 % of cement goes into applications where higher carbonation raises no durability concern. So the mitigation product extends life in unreinforced and sheltered uses and can shorten it in outdoor reinforced structures. NOTE: an earlier version also claimed a reflective/“cool surface” urban-heat benefit; neither cited source addresses albedo or urban heat islands, so that limb has been removed pending a source. |
| What it implies for policy | Standards and public procurement can reward the mixes that deliver both lower embodied carbon and demonstrated durability/albedo. |
| Literature | - Sharifi, A. (2021) "Co-benefits and synergies between urban climate change mitigation and adaptation measures: A literature review", Science of the Total Environment 750:141642. https://doi.org/10.1016/j.scitotenv.2020.141642 - Habert, G. et al. (2020) Nature Reviews Earth & Environment 1:559–573. https://doi.org/10.1038/s43017-020-0093-3 |

Table 6	Cement CCS adds a water penalty in water-stressed basins

| Trade-off | Cement CCS adds a water penalty in water-stressed basins |
| --- | --- |
| Mitigation lever | Post-combustion carbon capture on cement plants — the indispensable lever for the sector’s process emissions. |
| Adaptation dimension | Freshwater availability for capture solvents and cooling under drought. |
| Mechanism | Carbon capture raises a plant’s water intensity — retrofitting post-combustion capture raises a coal plant’s water intensity by roughly 55%, and CCS water footprints span orders of magnitude (≈0.7–575 m³ per tCO₂) depending on the CCS technology — the 575 upper bound is BECCS including biomass evapotranspiration, while post-combustion capture, the cement-relevant case, sits at roughly 2.2–2.6 m³/tCO₂ (Rosa et al. cover power plants and DACCS/BECCS, not cement, so the transfer to kilns is an inference). Cement CCS sited in drought-prone basins therefore adds water demand exactly where EUCRA sees scarcity rising; cooling choices (dry/hybrid) and reuse determine how sharp the conflict becomes. |
| What it implies for policy | CCS permitting and Net-Zero Industry Act support for cement should carry the same water-stress screen proposed for hydrogen, with capture/cooling configurations sized to local hydrology. |
| Literature | - Rosa, L., Sanchez, D.L., Realmonte, G., Baldocchi, D. & D’Odorico, P. (2021) "The water footprint of carbon capture and storage technologies", Renewable and Sustainable Energy Reviews 138:110511. https://doi.org/10.1016/j.rser.2020.110511 - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment |

## Chemicals & petrochemicals

Table 7	Bio-based feedstock switching competes with land and water

| Trade-off | Bio-based feedstock switching competes with land and water |
| --- | --- |
| Mitigation lever | Replacing fossil feedstocks with biomass-derived feedstocks for chemicals/plastics. |
| Adaptation dimension | Food-system resilience and land/water availability under climate stress. |
| Mechanism | Large-scale biomass feedstocks for chemicals add to the same land- and water-competition pressures the report flags for biofuels and bioenergy: they can raise vulnerability of food and ecosystems in a drying climate and drive indirect land-use change. The trade-off is smallest for waste/residue and CO₂-based (CCU) feedstocks. |
| What it implies for policy | Feedstock-switching incentives should prioritise circular carbon and residues over primary biomass, mirroring the report’s cascading-use logic. |
| Literature | - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ - IPCC (2019) "Climate Change and Land" (SRCCL), SPM on land-based mitigation competition. https://www.ipcc.ch/srccl/ |

Table 8	Riverside/coastal clusters: electrified but climate-exposed

| Conditional | Riverside/coastal clusters: electrified but climate-exposed |
| --- | --- |
| Mitigation lever | Electrification and heat integration of large coastal/riverside chemical clusters. |
| Adaptation dimension | Exposure to river low-flows, cooling-water limits, sea-level rise and storm surge. |
| Mechanism | The big integrated chemical parks (e.g. along the Rhine–Scheldt axis) benefit from grid and hydrogen infrastructure for electrification, but sit exactly where cooling-water availability falls in droughts and where flood/surge risk rises. The same site that enables deep mitigation carries concentrated climate risk, so the two agendas must be planned on the same asset. |
| What it implies for policy | Cluster decarbonisation roadmaps (and their permitting) should embed climate-proofing of cooling water and flood defences, not treat them separately. |
| Literature | - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment - IPCC (2022) AR6 WGII, "Impacts, Adaptation and Vulnerability" — Ch. 13 (Europe) & Ch. 18 (Climate Resilient Development Pathways). https://www.ipcc.ch/report/ar6/wg2/ |

## Cross-cutting — circularity & material efficiency

Table 9	Industrial electrification shifts risk onto a climate-exposed grid

| Conditional | Industrial electrification shifts risk onto a climate-exposed grid |
| --- | --- |
| Mitigation lever | Electrifying industrial heat and processes. |
| Adaptation dimension | Grid reliability under heatwaves, droughts (hydro/thermal derating) and storms. |
| Mechanism | Electrification is central to industrial mitigation but concentrates the sector’s resilience on the power system, which itself is stressed by climate: heatwaves cut thermal-plant output and transmission capacity, droughts cut hydro and cooling, and demand for cooling peaks at the same time. Deep electrification is only a robust strategy if paired with a climate-resilient, flexible grid. |
| What it implies for policy | The report’s "plan and operate the energy system as a whole" gap is also an adaptation gap; industrial electrification and grid climate-proofing are one problem. |
| Literature | - van Vliet, M.T.H. et al. (2016) "Power-generation system vulnerability and adaptation to changes in climate and water resources", Nature Climate Change 6:375–380. https://doi.org/10.1038/nclimate2903 - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ |

Table 10	Material efficiency lowers emissions and critical-material exposure

| Synergy | Material efficiency lowers emissions and critical-material exposure |
| --- | --- |
| Mitigation lever | Doing more with less material (light-weighting, reuse, longer product life). |
| Adaptation dimension | Reduced exposure to climate- and geopolitics-disrupted primary-material and critical-raw-material supply chains. |
| Mechanism | Cutting primary material throughput reduces both embodied emissions and the volume of vulnerable imports the EU must secure, so material efficiency doubles as a supply-security/resilience measure. This is the same logic the report applies when it criticises the recycling-only focus of the first Circular Economy Action Plan. |
| What it implies for policy | Material-efficiency mandates (Ecodesign for circularity, product-lifetime rules) should be credited for their resilience value, not only their tonnes of CO₂. |
| Literature | - IEA (2021) "The Role of Critical Minerals in Clean Energy Transitions". https://www.iea.org/reports/the-role-of-critical-minerals-in-clean-energy-transitions - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ |

# Transport

Cars and vans, heavy-duty road freight, rail, aviation, maritime, and the demand and modal-shift layer — where the network that has to carry fewer emissions also has to withstand more heat, more water and more disruption.

## Road passenger (cars & vans)

Table 11	Modal shift to active/public travel is also a health-resilience gain

| Synergy | Modal shift to active/public travel is also a health-resilience gain |
| --- | --- |
| Mitigation lever | Shifting car trips to walking, cycling and public transport in compact cities. |
| Adaptation dimension | Lower exposure to air pollution and heat, and reduced pressure on infrastructure. |
| Mechanism | Avoiding car kilometres cuts CO₂ while the accompanying urban form — compactness, green corridors, shade — reduces heat exposure and pollution and builds the classic mitigation–adaptation co-benefit documented for cities. Street trees and shading that make walking/cycling viable in heat are adaptation and mitigation at once. |
| What it implies for policy | Demand-moderation and modal-shift measures (which the report flags as under-used in EU strategy) should be evaluated for their heat- and health-resilience co-benefits. |
| Literature | - Sharifi, A. (2021) "Co-benefits and synergies between urban climate change mitigation and adaptation measures: A literature review", Science of the Total Environment 750:141642. https://doi.org/10.1016/j.scitotenv.2020.141642 - IPCC (2022) AR6 WGII, "Impacts, Adaptation and Vulnerability" — Ch. 13 (Europe) & Ch. 18 (Climate Resilient Development Pathways). https://www.ipcc.ch/report/ar6/wg2/ |

Table 12	EV batteries, charging and minerals are climate-exposed

| Trade-off | EV batteries, charging and minerals are climate-exposed |
| --- | --- |
| Mitigation lever | Battery-electric vehicles and charging infrastructure. |
| Adaptation dimension | Battery performance and charging reliability under extreme heat and flooding; mineral supply-chain exposure. |
| Mechanism | The battery supply chain, not the vehicle, is where the cited evidence sits. Lithium, cobalt, copper and rare earths are concentrated in climate-exposed places — around 80 % of Chilean copper comes from arid, water-stressed areas, and the 2022 Sichuan drought cut lithium output by roughly 1 200 t — so a fleet-wide switch to EVs deepens EU exposure to climate-driven supply shocks on top of an already concentrated market (~98 % rare-earth dependence on China). NOTE: an earlier version of this entry also claimed extreme heat degrades battery range and lifetime and floods kerbside chargers; neither cited source supports that (the full EUCRA text does not mention charging at all), so it has been removed pending a source. |
| What it implies for policy | Charging-network and battery standards should include thermal/flood resilience, and vehicle-efficiency incentives (the report’s ambition gap) also cut mineral exposure. |
| Literature | - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment - IEA (2021) "The Role of Critical Minerals in Clean Energy Transitions". https://www.iea.org/reports/the-role-of-critical-minerals-in-clean-energy-transitions |

Table 13	EV fleets as distributed storage can back up a climate-stressed grid

| Conditional | EV fleets as distributed storage can back up a climate-stressed grid |
| --- | --- |
| Mitigation lever | Electrifying cars and vans with smart charging and vehicle-to-grid (V2G) capability. |
| Adaptation dimension | Power-system flexibility and reserve capacity during heatwave demand peaks and climate-driven supply shocks. |
| Mechanism | A large EV fleet is also a large distributed battery: with smart charging and V2G it can shift load away from heatwave-driven peaks and provide reserve when extremes derate thermal, hydro or network capacity — turning the electrification lever into a resilience resource instead of only an added load. The benefit is conditional on chargers, market rules and aggregation actually enabling bidirectional participation. |
| What it implies for policy | AFIR roll-out and network codes should enable bidirectional charging and aggregation so the EV fleet contributes to, rather than burdens, system adequacy in extremes — the flip side of this note’s EV-exposure trade-off. |
| Literature | - Kempton, W. & Tomić, J. (2005) "Vehicle-to-grid power implementation: From stabilizing the grid to supporting large-scale renewable energy", Journal of Power Sources 144(1):280–294. https://doi.org/10.1016/j.jpowsour.2004.12.022 - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ |

## Road freight (heavy-duty)

Table 14	Road→rail freight shift depends on a climate-proofed railway

| Conditional | Road→rail freight shift depends on a climate-proofed railway |
| --- | --- |
| Mitigation lever | Shifting freight from heavy trucks to (electrified) rail and inland waterways. |
| Adaptation dimension | Resilience of the receiving rail/waterway network to heat, floods and low river levels. |
| Mechanism | Modal shift is a core mitigation lever, but the lower-carbon modes it relies on are themselves climate-sensitive: rail to heat-buckling and flooding, inland waterways to droughts. The shift is a durable win only if the target network is adapted; otherwise mitigation gains are eroded by reliability losses during extremes. |
| What it implies for policy | The report’s modal-shift ambition/implementation gaps (Combined Transport Directive, Rail Freight Corridors, TEN-T) should be read together with TEN-T climate-proofing requirements. |
| Literature | - Palin, E.J., Stipanovic Oslakovic, I., Gavin, K. & Quinn, A. (2021) "Implications of climate change for railway infrastructure", WIREs Climate Change 12(5):e728. https://doi.org/10.1002/wcc.728 - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment |

## Rail

Table 15	The low-carbon rail backbone is itself heat- and flood-vulnerable

| Trade-off | The low-carbon rail backbone is itself heat- and flood-vulnerable |
| --- | --- |
| Mitigation lever | Electrified rail as the low-carbon spine of passenger and freight transport. |
| Adaptation dimension | Track buckling and speed restrictions in heatwaves; line flooding, scour and landslides. |
| Mechanism | Rail is among the lowest-carbon modes, yet steel track buckles and overhead lines sag in extreme heat (forcing speed restrictions), and lines flood or wash out in extreme rain. Rising heatwave frequency in Europe therefore threatens the reliability of the very asset the mitigation strategy leans on — protecting it is a precondition, not an optional add-on. |
| What it implies for policy | Rail electrification funding should be paired with heat-/flood-proofing (rail stress management, drainage) so adaptation protects the mitigation investment. |
| Literature | - Dobney, K., Baker, C.J., Quinn, A.D. & Chapman, L. (2009) "Quantifying the effects of high summer temperatures due to climate change on buckling and rail related delays in south-east United Kingdom", Meteorological Applications 16(2):245–251. https://doi.org/10.1002/met.114 - Palin, E.J., Stipanovic Oslakovic, I., Gavin, K. & Quinn, A. (2021) "Implications of climate change for railway infrastructure", WIREs Climate Change 12(5):e728. https://doi.org/10.1002/wcc.728 - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment |

## Aviation

Table 16	Biomass-based SAF competes with land, water and food resilience

| Trade-off | Biomass-based SAF competes with land, water and food resilience |
| --- | --- |
| Mitigation lever | Sustainable aviation fuels (SAF) mandated under ReFuelEU Aviation. |
| Adaptation dimension | Land/water competition and food-system resilience under climate stress. |
| Mechanism | ReFuelEU Aviation (Art. 4(5)) excludes food- and feed-crop biofuels from the SAF mandate, but the eligible biomass pathways (RED Annex IX feedstocks, incl. certain intermediate crops added by Delegated Directive 2024/1405) still draw on a finite biomass, land and water base — the biofuel indirect-effects concern the report raises — so scaling them can erode food- and ecosystem resilience in a drying climate. The trade-off is smallest for genuine wastes/residues; synthetic e-SAF instead shifts the pressure onto renewable electricity and water. |
| What it implies for policy | SAF sub-targets should steer hard toward genuine residues and e-fuels; the report’s biofuel ambition/implementation gaps apply directly here. |
| Literature | - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ - Staples, M.D. et al. (2018) "Aviation CO₂ emissions reductions from the use of alternative jet fuels", Energy Policy 114:342–354. https://doi.org/10.1016/j.enpol.2017.12.007 |

Table 17	Heat cuts aircraft payload; coastal airports face rising seas

| Trade-off | Heat cuts aircraft payload; coastal airports face rising seas |
| --- | --- |
| Mitigation lever | Efficiency and demand moderation in aviation. |
| Adaptation dimension | Higher temperatures reduce lift/payload; low-lying airports face sea-level rise and surge. |
| Mechanism | Higher air temperature lowers air density, so a fully loaded aircraft needs more runway or must shed payload — Coffel et al. find 10–30 % of departures at the daily maximum temperature weight-restricted, shedding 0.5–4 % of payload and fuel on average. The effect is strongly site-specific and bites hardest at hot, high-elevation or short-runway airports (LaGuardia ~50 % of departures, Dubai ~55 %); the same study names London Heathrow and Paris Charles de Gaulle among the airports MINIMALLY affected, so the European exposure runs mainly through coastal siting rather than heat. On that, EUCRA is explicit: airports and harbours in low-elevation coastal areas are at risk from sea-level rise. Efficiency losses act against the sector's mitigation effort at exactly the times demand peaks. |
| What it implies for policy | Airport adaptation planning belongs alongside aviation decarbonisation and the report’s call to close the extra-EU ETS exemption. |
| Literature | - Coffel, E. & Horton, R. (2015) "Climate Change and the Impact of Extreme Temperatures on Aviation", Weather, Climate, and Society 7(1):94–102. https://doi.org/10.1175/WCAS-D-14-00026.1 - Coffel, E.D., Thompson, T.R. & Horton, R.M. (2017) "The impacts of rising temperatures on aircraft takeoff performance", Climatic Change 144:381–388. https://doi.org/10.1007/s10584-017-2018-9 - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment |

## Maritime & inland waterways

Table 18	Green-fuel bunkering is being built at the most sea-exposed nodes

| Trade-off | Green-fuel bunkering is being built at the most sea-exposed nodes |
| --- | --- |
| Mitigation lever | Alternative marine fuels (e-ammonia, e-methanol) and shore-side electricity at ports. |
| Adaptation dimension | Ports’ acute exposure to sea-level rise, storm surge and extreme waves. |
| Mechanism | The fuel-bunkering, shore-power and hydrogen/ammonia infrastructure needed to decarbonise shipping is concentrated in ports — precisely the assets most exposed to sea-level rise and storm surge. New low-carbon port infrastructure that is not designed for those hazards risks being stranded or damaged, undercutting the mitigation investment. |
| What it implies for policy | FuelEU Maritime / AFIR port-infrastructure roll-out should be conditioned on climate-proofing of the port itself. |
| Literature | - Izaguirre, C., Losada, I.J., Camus, P., Vigh, J.L. & Stenek, V. (2021) "Climate change risk to global port operations", Nature Climate Change 11:14–20. https://doi.org/10.1038/s41558-020-00937-z - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment - IPCC (2022) AR6 WGII, "Impacts, Adaptation and Vulnerability" — Ch. 13 (Europe) & Ch. 18 (Climate Resilient Development Pathways). https://www.ipcc.ch/report/ar6/wg2/ |

Table 19	Droughts strand the low-carbon inland-waterway option

| Trade-off | Droughts strand the low-carbon inland-waterway option |
| --- | --- |
| Mitigation lever | Shifting freight to inland waterways (a low-carbon mode). |
| Adaptation dimension | Falling and more variable river levels in droughts (e.g. Rhine 2018, 2022). |
| Mechanism | Inland shipping is low-carbon per tonne-km but directly hostage to river hydrology: the 2018 and 2022 Rhine low-water episodes forced barges to sail part-loaded (at a 78 cm gauge at Kaub, roughly four times as many vessels are needed as at 250 cm). Crucially, the loss is NOT absorbed by other modes — Ademmer et al. find no evidence of a substantial shift to road or rail, so the disruption propagates into industrial output: about 0.034 % lower German industrial production per low-water day, ~1 % in a month with 30 such days, peaking at −1.5 % (≈ −0.4 % of GDP) in November 2018. Climate change degrades the reliability of the very mode the mitigation strategy wants to grow, and the alternatives do not take up the slack. (The 2018/2022 episodes are documented by EUCRA; Ademmer et al.’s own sample ends in March 2019.) |
| What it implies for policy | Modal-shift-to-water targets need a hydrological-risk hedge (fleet design for low draught, redundancy with rail) or they will underperform in dry years. |
| Literature | - Ademmer, M., Jannsen, N. & Meuchelböck, S. (2023) "Extreme Weather Events and Economic Activity: The Case of Low Water Levels on the Rhine River", German Economic Review 24(2):121–144. https://doi.org/10.1515/ger-2022-0077 - EEA (2024) "European Climate Risk Assessment" (EUCRA), EEA Report 01/2024. https://www.eea.europa.eu/publications/european-climate-risk-assessment |

## Cross-cutting — demand & modal shift

Table 20	Avoiding travel demand reduces both emissions and exposure

| Synergy | Avoiding travel demand reduces both emissions and exposure |
| --- | --- |
| Mitigation lever | Transport-demand moderation, teleworking and compact settlement patterns. |
| Adaptation dimension | Less exposure of people and goods to disrupted, climate-stressed networks. |
| Mechanism | Reducing the need to travel cuts emissions and simultaneously reduces the population and freight exposed to network disruptions (heat, floods) — the least infrastructure is also the least vulnerable infrastructure. This is the "avoid" tier of avoid–shift–improve doing double duty. |
| What it implies for policy | The report’s finding that demand moderation is absent from EU mobility strategy is thus both a mitigation and a resilience gap. |
| Literature | - IPCC (2022) AR6 WGIII, "Mitigation of Climate Change" — Ch. 10 (Transport) & Ch. 11 (Industry). https://www.ipcc.ch/report/ar6/wg3/ - Sharifi, A. (2021) "Co-benefits and synergies between urban climate change mitigation and adaptation measures: A literature review", Science of the Total Environment 750:141642. https://doi.org/10.1016/j.scitotenv.2020.141642 |

Table 21	Climate-proofing transport infrastructure has an embodied-carbon cost

| Trade-off | Climate-proofing transport infrastructure has an embodied-carbon cost |
| --- | --- |
| Mitigation lever | Keeping transport-infrastructure embodied emissions down. |
| Adaptation dimension | Hardening roads, bridges and drainage against heat and extreme rainfall. |
| Mechanism | Adapting transport assets — heavier drainage, reinforced bridges, raised roads, heat-resistant pavements — is material- and cement-intensive, so the adaptation programme pulls against the embodied-carbon side of the mitigation ledger, echoing the cement/adaptation-demand trade-off in the industry chapter. |
| What it implies for policy | Resilience upgrades should specify low-carbon and nature-based solutions (e.g. sustainable drainage) to blunt the embodied-carbon penalty. |
| Literature | - Sharifi, A. (2020) "Trade-offs and conflicts between urban climate change mitigation and adaptation measures: A literature review", Journal of Cleaner Production 276:122813. https://doi.org/10.1016/j.jclepro.2020.122813 - IPCC (2022) AR6 WGII, "Impacts, Adaptation and Vulnerability" — Ch. 13 (Europe) & Ch. 18 (Climate Resilient Development Pathways). https://www.ipcc.ch/report/ar6/wg2/ |

# Taking this forward

Three things this note is ready to be used for in the next report cycle.

Check a policy proposal against the trade-offs

The trade-off entries are the ones with drafting consequences: each names a design choice that decides whether a mitigation instrument raises or lowers climate vulnerability. Where a proposal touches one of them — hydrogen siting under water stress, biomass supply under drought, thermal and hydrological limits on rail and inland waterways — the entry's “what it implies for policy” row is the test to apply.

Feed the conditional entries into the evidence plan

A conditional tag means the sign of the interaction depends on design, siting or resource context, and that the literature does not settle it. Those are the entries worth commissioning or collecting evidence on before the chapter drafting starts.

Carry the subsector structure across

Because the subsectors mirror the report's own, an entry can be moved into the corresponding chapter section without re-cutting the analysis — and it lines up with the policy-gap note for transport and industry, which uses the same subsector taxonomy.

# Sources and provenance

What each part of this note rests on, and what still needs a human sign-off.

Literature. Every entry cites the primary work behind the mechanism it describes. The recurring sources are IPCC AR6 WGII (Ch. 13 Europe, Ch. 18) and WGIII (Ch. 10 Transport, Ch. 11 Industry), the EEA European Climate Risk Assessment (EUCRA, 2024), and the peer-reviewed studies listed per entry.

Citation check. On 10 July 2026 every citation was verified against Crossref and publisher records — titles, authors, journals, volumes, DOIs, and the claim attributed to each source. That pass corrected a wrong DOI, a dead publisher link, an incomplete citation and a publication date.

Pending verification. The framing of each interaction and its assignment to a subsector are working judgements compiled for this prep cycle. They need the sector lead's sign-off before any of this is used in a Board product.

Live version. This note is the offline copy of the Method Hub module Summer Prep › Policy Gap 2.0 Report › Synergies & Trade-offs, which is generated from src/data/summer-prep-synergies.ts. If the two differ, the Hub is the current one.

Report. ESABCC (2024) “Towards EU climate neutrality: Progress, policy gaps and opportunities”, January 2024. https://climate-advisory-board.europa.eu/reports-and-publications/towards-eu-climate-neutrality-progress-policy-gaps-and-opportunities